Technical data:

Bio Uro Clean Treated Waste Water Results:

| Parameter | Units |
| --- | --- |
| Biochemical oxygen demand (BOD 5) | <10 mg/l |
| Total suspended solid @ 103°C | 5 mg |
| Ammonia (NH3-N) | 0.06 mg/l |
| Nitrate (NO3-N) | 5.4 mg/l |
| Total phosphorus (P) | 7.2 mg/l |
| pH ( Electrometric Determination) | 6.9 |

Bio Uro Clean system – Basic information.

Treatment capacity 				4080 litres

Primary chamber 	treatment capacity		2040 litres

Treatment chamber capacity			2040 litres

Design population				6 PE

Chamber Diameter				1170 mm

Overall length					2730 mm

Total Height					2070 mm

Weight (empty)					2000 kg

Design flow rate					1200 litres/day

BOD load					0.36 kg/day

Inlet invert to base				1890 mm

Outlet invert to base				1890 mm

Ground level to inlet/outlet invert level		600 mm

Compressor motor rating				0.49 amps

Retention time (based on design flow)		80 hours

# Minimum separation distances

The unit should be sited so that adequate access is available for safe installation, maintenance and desludging of the unit. The minimum separation distances below apply.

| Separation distances (m) | Unit | Irrigation Area |
| --- | --- | --- |
| Dwelling served | 7 | 10 |
| Adjacent dwelling | 7 | 10 |
| Wall | 3.5 | 3 |
| Road | 4 | 4 |
| Site boundary | 3.5 | 3 |
| Potable water source | 10 | 30 - 100 |

Copyright Bio Uro Clean 2007 | www.biouroclean.ie